# AS 1288 Glass Thickness Calculator — Project Summary

## Duce Timber Windows and Doors

### Version: V1.0 (In Development)

### Last Updated: June 2026

---

## 1. Project Overview

An internal desktop calculator tool for determining minimum glass thickness and checking glass compliance under wind loads in accordance with AS 1288. Built from scratch for Duce Timber Windows and Doors. The tool covers wind load calculations only (AS 1288 Section 4). Human impact considerations are out of scope for V1.0.

---

## 2. Technical Stack

- **Language:** Python 3.14

- **Core libraries:** pandas

- **GUI framework:** tkinter (Phase 2 — not yet built)

- **Packaging:** PyInstaller EXE (Phase 2 — not yet built)

- **Future:** Flask web app (Phase 3 — not yet started)

---

## 3. Project Folder Structure

```

duce_glass_calc/

├── data/

│   ├── Wind_Load_Check_Tables_Full.csv

│   ├── N_C_Tables.csv

│   └── Table_4_1_Minimum_Glass_Thickness.csv

├── outputs/

├── src/

│   ├── calculator.py        ← Core calculation engine (Phase 1 complete)

│   └── test_runner.py       ← Automated validation test runner

└── [this document]

```

---

## 4. Data Files

### Wind_Load_Check_Tables_Full.csv

- 315 rows, 10 columns

- Contains ULS and SLS wind load coefficients (k1, k2, k3, k4) from AS 1288

- Columns: Wind Load Type, Glass Type, Glass sub-type 1, Nominal Thickness (mm), Support Condition, AR, k1, k2, k3, k4

- ULS rows: glass-type specific (Monolithic Annealed, Toughened, Heat-strengthened, Laminated Annealed)

- SLS rows: Glass Type = All, applies to all glass types

### N_C_Tables.csv

- 20 rows, 4 columns

- Wind classification pressure lookup for AS 4055

- Columns: Rating, Pressure Type, General, Corner

- Ratings: N1–N6, C1–C4

- Pressures in Pa for ULS and SLS

### Table_4_1_Minimum_Glass_Thickness.csv

- 18 rows, 3 columns

- AS 1288 Table 4.1 — converts actual measured thickness to nominal thickness

- Columns: Glass Type, Nominal Thickness (mm), Minimum Thickness (mm)

- Glass types: Monolithic, Laminated (subtype not relevant for this lookup)

---

## 5. Engineering Logic

### Calculation Modes

**Mode 1 — Minimum Thickness Finder**

Finds the minimum compliant glass thickness for a selected glass type and glazing configuration. Used when glass has not yet been specified.

**Mode 2 — Compliance Checker**

Checks whether a glass of known actual thickness meets wind load requirements. Used when glass has already been specified and needs to be verified. Uses AS 1288 Table 4.1 to convert actual thickness to nominal thickness before lookup.

---

### Inputs (both modes)

| Input | Description |

|---|---|

| Height (mm) | Panel height |

| Width (mm) | Panel width |

| Support condition | 4-edge or 2-edge |

| Span dimension | Height or Width (2-edge only — dimension between supported edges) |

| Wind load method | Direct pressure (AS 1170.2) or N/C rating (AS 4055) |

| ULS pressure (Pa) | Ultimate limit state wind pressure |

| SLS pressure (Pa) | Serviceability limit state wind pressure |

| N/C Rating | N1–N6 or C1–C4 (if N/C method selected) |

| Window location | General or Corner (if N/C method selected) |

| Glass type(s) | One or more of the four available types |

| Glazing config | Single, IGU Double, IGU Triple |

**Mode 2 additional inputs (per pane):**

- Glass type and subtype for each pane

- Actual measured thickness for each pane (mm)

- Panes labelled: Single / Outer + Inner / Outer + Middle + Inner

---

### Span and Aspect Ratio Rules

- **AR** = longest dimension ÷ shortest dimension (always ≥ 1.0)

- **4-edge:** Span = shortest dimension. AR rounded up to next table value (1, 1.25, 1.5, 1.75, 2, 2.5, 3, 5)

- **2-edge:** Span = dimension between supported edges (user confirmed). AR = Independent (single set of k values)

- **AR > 5.0:** Out of scope. Display message: *"Glass size outside scope of AS 1288 Section 4. Requires engineering for Performance Solution."*

---

### ULS Formula (AS 1288)

$$B = k_1 \times (P_u + k_2)^{k_3} + k_4$$

- B = maximum allowable span (mm)

- Pu = ULS wind pressure (kPa) — convert from Pa by dividing by 1000

- k1–k4 = coefficients from wind load table

**Pass condition:** actual span ≤ B

---

### SLS Formula (AS 1288)

$$B = t \times [k_1 \times (P_s + k_2)^{k_3} + k_4]$$

- t = nominal glass thickness (mm)

- Ps = SLS wind pressure (kPa) — convert from Pa by dividing by 1000

- SLS k values apply to all glass types (Glass Type = All in CSV)

**Pass condition:** actual span ≤ B

---

### Mode 1 Calculation Sequence

1. Calculate AR and span from panel dimensions

2. Check AR scope (reject if > 5.0)

3. For each glass type selected, starting at thinnest thickness:

- Apply k_pane to get effective pressures (see IGU section below)

- Run ULS check — find minimum passing thickness

- Run SLS check starting from ULS minimum — find minimum passing thickness

- Final minimum = SLS minimum (always ≥ ULS minimum)

4. Report ULS minimum, SLS minimum, and final governing minimum per glass type

---

### Mode 2 Calculation Sequence

1. Calculate AR and span from panel dimensions

2. Check AR scope

3. For each pane:

- Convert actual thickness to nominal using Table 4.1

- Calculate k_pane using actual thicknesses of all panes

- Apply k_pane to get effective ULS and SLS pressures

- Look up k values using nominal thickness

- Calculate B_uls and B_sls

- Report PASS or FAIL for ULS, SLS, and overall

---

### IGU Load Sharing (k_pane)

**Mode 1 — Equal thickness assumption:**

$$k_{pane} = \frac{1.25}{n}$$

Where n = number of panes. Result is a fixed constant:

- Single: k_pane = 1.0

- Double: k_pane = 0.625

- Triple: k_pane = 0.4167

**Mode 2 — Actual thicknesses:**

$$k_{pane} = \min\left(\frac{1.25 \times t_{pane}^3}{\sum t_i^3}, 1.0\right)$$

Capped at 1.0. Uses actual measured thicknesses for all panes.

---

### Glass Types and Available Nominal Thicknesses

| Glass Type | Sub-type | Nominal Thicknesses (mm) |

|---|---|---|

| Monolithic | Annealed | 3, 4, 5, 6, 8, 10, 12, 15, 19, 25 |

| Monolithic | Toughened | 4, 5, 6, 8, 10, 12, 15, 19, 25 |

| Monolithic | Heat-strengthened | 3, 4, 5, 6, 8, 10, 12 |

| Laminated | Annealed | 5, 6, 8, 10, 12, 16, 20, 24 |

---

## 6. Key Validation Rules

- AR string matching: table AR values stored as '1', '1.25' etc (not '1.0', '1.25'). Code strips trailing zeros before string comparison.

- Nominal thickness lookup uses broad glass type only (Monolithic or Laminated) — subtype is not relevant for Table 4.1

- N/C pressure lookup uses exact rating string match (e.g. 'N3', 'C1')

- k_pane cap: if calculated k_pane > 1.0 it is set to 1.0

---

## 7. Validated Test Cases

20 test cases validated manually against AS 1288 and confirmed matching via automated test runner (test_runner.py). All 20 cases pass. Test cases cover:

- All four glass types

- 4-edge and 2-edge support conditions

- AR at exact table values, between table values, and > 5.0 (out of scope)

- Direct pressure input and N/C rating input (General and Corner)

- Single, Double IGU, and Triple IGU glazing configurations

- Mode 2 with nominal thickness classification

- Mode 2 with mixed glass types in an IGU

---

## 8. Pending Features (Phase 2 — GUI)

- Desktop GUI using tkinter

- Two-tab or toggle interface for Mode 1 and Mode 2

- Dropdowns for glass type, support condition, glazing configuration

- Wind load input method selector (direct pressure vs N/C rating)

- Results panel showing ULS minimum, SLS minimum, final governing thickness

- Stepwise calculation report — generated on user demand

- Export report as TXT file

- Package as standalone EXE using PyInstaller

---

## 9. Future Versions

### V1.0 Notes

- All results display note: *"Human Impact requirements have not been considered. Results are for Wind Load resistance only (AS 1288 Section 4)."*

- Wind load input in Pa (converted to kPa internally before formula)

### V2.0 — Human Impact (not started)

- Human impact considerations per AS 1288

- Will override minimum thickness results from wind load check

- Will restrict which glass types can be used in certain applications

- Will be integrated into the same tool — results from V1.0 wind load engine feed into V2.0 human impact check

### Phase 3 — Flask Web App (not started)

- Same calculation logic migrated to Flask backend

- HTML frontend

- To be started after V1.0 EXE is validated and in use

---

## 10. How to Resume This Project in a New Conversation

Paste this document at the start of the new conversation with the following message:

*"I am continuing development of the AS 1288 Glass Thickness Calculator for Duce Timber Windows and Doors. The attached document summarises the project state. Phase 1 (calculation engine) is complete and validated. We are now working on [current phase]. Here is where we left off: [brief description]."*

---

*End of document*